# 6. Results and Discussion

The three neural forecasters and the LSTM-TCN ensemble were evaluated on an identical, leakage-safe test set comprising 2,865 forecast origins in September 2025 and 45,840 quarter-hour target values over a four-hour horizon. Model selection and any residual calibration used August data only; September remained untouched until final evaluation. Two input representations were compared at the same forecast origins: an unrestricted 72-hour history (288 quarter-hour steps) and a manually defined 42-hour representation (168 steps) motivated by the proposed Terneuzen-to-Ghent transport delay. Consequently, the differences below reflect representation and model behavior rather than unequal test windows.

## 6.1 Overall Matched-Window Performance

Table 1 summarizes the held-out performance of the LSTM, TCN, Chronos-2 foundation model, and the validation-prespecified equal-weight LSTM-TCN ensemble. Every configuration achieved positive RMSE skill relative to persistence. The manual 42-hour TCN obtained the lowest overall RMSE (0.1998 mS/cm), the highest all-horizon R2 (0.5138), and 10.95% RMSE skill. Calibrated Chronos-2 with the same representation was effectively co-leading (RMSE 0.2008 mS/cm; R2 0.5087; skill 10.48%). The best unrestricted model was calibrated Chronos-2 (RMSE 0.2054 mS/cm; skill 8.45%), establishing that the foundation model extracted useful structure from the full 72-hour context without task-specific weight updates.

Table 1. Held-out September performance across all four-hour forecast trajectories.

| Model | Input | RMSE (mS/cm) | R2 | RMSE skill (%) | 4 h skill (%) |
| --- | --- | --- | --- | --- | --- |
| LSTM | 72 h | 0.2117 | 0.4541 | 5.64 | 6.82 |
| LSTM | 42 h | 0.2110 | 0.4578 | 5.96 | 6.86 |
| TCN | 72 h | 0.2115 | 0.4553 | 5.74 | 6.93 |
| TCN | 42 h | 0.1998 | 0.5138 | 10.95 | 12.05 |
| LSTM-TCN ensemble | 72 h | 0.2106 | 0.4598 | 6.13 | 7.33 |
| LSTM-TCN ensemble | 42 h | 0.2041 | 0.4927 | 9.03 | 10.04 |
| Chronos-2, calibrated | 72 h | 0.2054 | 0.4861 | 8.45 | 9.34 |
| Chronos-2, calibrated | 42 h | 0.2008 | 0.5087 | 10.48 | 12.99 |

The model ranking was close enough to discourage a definitive winner claim. TCN and calibrated Chronos-2 differed by only 0.0010 mS/cm under the 42-hour representation, while the ensemble traded a small amount of point accuracy for a combination intended to reduce model-specific variance and fixed before the test was opened. Importantly, August validation selected the ensemble in both representation regimes, whereas the manual 42-hour TCN ranked first in September. This validation-test reversal indicates that small numerical differences among the leading models may not be stable across hydrological periods.

## 6.2 Horizon and Rapid-Change Behavior

Persistence remained a demanding baseline because conductivity is strongly autocorrelated, but the learned corrections became more valuable toward the four-hour endpoint. Table 2 shows that the manual 42-hour TCN and calibrated Chronos-2 reached 12.05% and 12.99% endpoint skill, respectively. The same configurations also retained substantial gains for the largest 10% of observed target changes. Thus, their advantage was not restricted to quiescent periods, although rapid-change trajectories remained intrinsically difficult.

Table 2. Endpoint and rapid-change diagnostics relative to persistence (higher skill is better).

| Model | Input | 4 h RMSE | 4 h skill (%) | Largest-change RMSE | Largest-change skill (%) |
| --- | --- | --- | --- | --- | --- |
| LSTM | 72 h | 0.2627 | 6.82 | 0.5363 | 8.31 |
| LSTM | 42 h | 0.2626 | 6.86 | 0.5149 | 11.96 |
| TCN | 72 h | 0.2624 | 6.93 | 0.5399 | 7.69 |
| TCN | 42 h | 0.2480 | 12.05 | 0.4961 | 15.17 |
| LSTM-TCN ensemble | 72 h | 0.2613 | 7.33 | 0.5371 | 8.18 |
| LSTM-TCN ensemble | 42 h | 0.2536 | 10.04 | 0.5046 | 13.72 |
| Chronos-2, calibrated | 72 h | 0.2556 | 9.34 | 0.5167 | 11.66 |
| Chronos-2, calibrated | 42 h | 0.2453 | 12.99 | 0.5011 | 14.32 |

Positive skill during large changes should not be confused with complete event reconstruction. R2 for the largest-change subset remained negative for every reported neural configuration (from -0.3092 for the manual 42-hour TCN to -0.5502 for the unrestricted TCN). The models reduced squared error relative to holding the latest value, yet still explained less event-to-event variance than would be needed for reliable plume-peak characterization. This result is consistent with smoothing of abrupt transitions and incomplete representation of external forcing.

## 6.3 Model-Specific Findings

### 6.3.1 Long Short-Term Memory Network

The residual LSTM contained 33,872 parameters for the unrestricted input and 33,552 for the 42-hour input. Its performance was almost representation-invariant: truncation reduced overall RMSE by only 0.0007 mS/cm (0.33%) and increased skill by 0.32 percentage points. Endpoint RMSE was virtually unchanged (0.2627 versus 0.2626 mS/cm). The unrestricted model stopped after 20 epochs and restored epoch 13, whereas the 42-hour model continued for all 40 epochs. The LSTM therefore provides little evidence that a hard 42-hour restriction is necessary for recurrent learning; gating appears able to suppress much of the irrelevant portion of the longer history.

### 6.3.2 Temporal Convolutional Network

The TCN contained 36,248 parameters for the unrestricted input and 36,040 for the manual input. Its dilated residual stack had a nominal receptive field of 505 quarter-hour steps, exceeding both input lengths and allowing full-window access. Unlike the LSTM, the TCN benefited strongly from the physical prior: the 42-hour representation lowered overall RMSE by 0.0117 mS/cm (5.53%), raised skill by 5.21 percentage points, and produced the best overall deterministic result. It also achieved the strongest rapid-change skill (15.17%). This sensitivity suggests that convolutional feature extraction was helped by removing temporally diffuse history and emphasizing the lag-informed channel structure.

### 6.3.3 Chronos-2 Foundation Model

Chronos-2 was evaluated zero-shot using its median forecast and then blended with persistence using weights estimated exclusively from August residuals; its 119,477,664 pretrained parameters were not fine-tuned. Raw zero-shot RMSE was 0.2061 mS/cm for 72 hours and 0.2037 mS/cm for 42 hours. Calibration improved these values to 0.2054 and 0.2008 mS/cm, respectively. The gain from calibration was therefore modest but consistent for overall RMSE, and the manual representation remained superior. Notably, the raw manual forecast slightly outperformed its calibrated version for the largest changes (RMSE 0.4943 versus 0.5011 mS/cm), showing that a calibration optimized for aggregate validation error can attenuate event-scale corrections.

### 6.3.4 LSTM-TCN Ensemble

The ensemble was the equal-weight mean of LSTM and TCN predictions and was specified before evaluating September. It improved upon either unrestricted component overall (RMSE 0.2106 mS/cm), confirming some error complementarity. Under the manual representation it achieved RMSE 0.2041 mS/cm and 9.03% skill, but it did not surpass the TCN because averaging diluted the TCN's stronger 42-hour signal. The ensemble remains attractive when ranking stability and low procedural complexity matter; however, these results do not support presenting it as the most accurate final model. The reported ensemble contains LSTM and TCN only and should not be interpreted as a three-model blend including Chronos-2.

## 6.4 Interpretation of the 42-Hour Hypothesis

The manually defined 42-hour method improved every neural forecaster in overall RMSE, but the magnitude was architecture-dependent: 0.33% for LSTM, 5.53% for TCN, 3.09% for the LSTM-TCN ensemble, and 2.22% for calibrated Chronos-2. At the four-hour endpoint, the reductions were 0.04%, 5.49%, 2.95%, and 4.03%, respectively. The consistent direction of change supports the 42-hour lag as a useful inductive bias, especially for TCN and Chronos-2, but does not establish a unique plume travel time. The manual and unrestricted representations differ in sequence length and feature construction as well as lag emphasis, so the experiment cannot attribute the gain solely to the Terneuzen observation at exactly t - 42 hours.

A stronger causal test would keep dimensionality and sensor availability fixed while comparing multiple predeclared lags (for example 24, 36, 42, 48, and 60 hours), include a temporally shuffled-lag control, and select the lag using training and validation data only. Repetition across seasons and discharge regimes is also required because advection time may vary with canal operations and freshwater inflow. The present finding should therefore be framed as evidence that a compact, physically informed temporal representation improves generalization, not as confirmation of an invariant 42-hour transport constant.

## 6.5 Probabilistic and Computational Considerations

Chronos-2 additionally supplied an 80% predictive interval. The unrestricted representation attained 75.52% empirical coverage overall and 79.90% coverage at four hours, with mean width 0.3027 mS/cm. The sharper manual representation reduced mean width to 0.2281 mS/cm but covered only 60.59% overall and 63.42% at four hours; its interval score also deteriorated from 0.6036 to 0.6419. Thus, the 42-hour representation improved point accuracy at the cost of substantial undercoverage. Operational deployment would require post-hoc interval calibration on blocked, representative periods rather than treating the native quantiles as calibrated uncertainty bounds.

Computationally, the compact input reduced Chronos-2 CPU inference time for the combined validation and test sets from 573.54 to 279.40 seconds, a 51.3% reduction. For the task-specific networks, the shorter TCN completed training in 200.4 seconds versus 249.9 seconds for the unrestricted model despite running more epochs; LSTM timing was dominated by its longer optimization path (308.2 versus 259.3 seconds). These measurements are hardware- and stopping-rule-specific, but they show that the compact representation can reduce sequence-processing cost without sacrificing accuracy.

## 6.6 Limitations and Operational Implications

All principal configurations beat persistence on the held-out trajectories, and gains generally increased at the longest operationally relevant lead time. The manual 42-hour TCN and calibrated Chronos-2 are therefore the strongest candidates for further evaluation: the former provides the best deterministic score with a compact trainable architecture, while the latter offers competitive zero-shot performance and probabilistic output. Their separation is small enough that model choice should also consider calibration, compute, maintainability, and stability across future monitoring periods.

Several limitations constrain generalization. The test set covers one month, and adjacent 15-minute forecast origins share most history and targets; the 2,865 rows are therefore not independent replicates for significance testing. Neural results were obtained from one fixed training run per configuration, so seed-to-seed uncertainty was not quantified. The inputs also omit potentially important forcings, including tide, discharge, rainfall, temperature, vessel movement, and lock operation. Future work should use blocked event- or day-level uncertainty estimates, repeated seeds, additional years and seasons, out-of-distribution events, and external hydrodynamic covariates. Until those tests are available, the three networks and ensemble should be regarded as a competitive group, with the manual 42-hour method retained as a useful but not yet causal physical prior.